[image: image1.wmf]
Gnoss/Marin County Airport







      415-897-2295

451 Airport Road








             Fax 415-893-1275

Novato, CA 94945-1419






     e-mail: daveward@airward.com

                                                                                                                                                   http://www.airward.com
RV-10 DOOR HINGE KIT


                                 [image: image2.jpg]



AirWard has developed a replacement to the original installation procedure for attaching the main doors to the canopy.  So much emphasis has been concentrated on the main door latch system and no consideration for the two hinges holding the main door to the canopy.  This 1/8” fiberglass with four countersunk screws is subject to quite a bit of tension when both latching the main door and when the door is in the up position in the wind.  To remove/install a door for maintenance is also very time consuming with the current system.  With our kit it requires removing the eight bolts from the canopy side of the hinge, without the problem of holding the nuts.  The inside of the door hinge cavity is now able to be fiberglassed over for a better finished look and an area for the door seal to contact. 

The kit consists of all the material necessary to complete all four hinge positions.  The only item we retain from the Van’s Kit is the hinge.  All the attaching screws are replaced with AN3 stainless bolts and bolted into anchor nuts mounted to reinforcing plates.  Total weight increase is less then 12 ounces.

Each hinge position consists of the following:

Two reinforcing aluminum outer plates;

Two reinforcing aluminum inner plates;

Four AN3C6A Bolts for the door attachment;

Four AN3C11A Bolts for the canopy attachment;

Eight NAS1149C0332R Thin Stainless Washers;

Eight MS21071L3 Anchor Nuts;

Sixteen MS20426AD3-4 Rivets.

Total Cost for the Four Hinge Kits is $349.00.

Free shipping in US and Canada.

[image: image3.jpg]


                    [image: image4.jpg]



[image: image5.jpg]


                   [image: image6.jpg]



[image: image7.jpg]


                  [image: image8.jpg]



