WING GAP SEAL

       See drawing sheet 44. In your kit are four 1/2"x5/8"x30" sheet metal angles. Clamp two of these together with approximately 0.032 gap between the 1/2" legs and drill #30 holes with 1" spacing starting from one end for 16". Then cut metal to each of the holes so channels can be contoured to the wing. 
       Drill and install channels with 2" spacing to IB side of IB wing ribs with 1/8" rivets. Lay 0.020 sheet metal on top of wings and mark along each channel (I think it is 6061 aluminum –thin, but kind of stiff). Add approximately 1/2" to each side of these marks and cut (You don’t have to get too tight of a fit here). 
       Now cut a section of 1‑112" tubing to fit between wing LE tubes. Hold tube in position and slide gap seal LE on tube and rivet in place. Cut a 1" long piece of 1‑1/2" aluminum tube. Then cut out 1/4" gap, as you did making rear spar splices. Slide this piece into wing LE tube with Epoxy applied and use sheet metal screws to attach the forward corners of gap seal to wing LE tubes. The doubler makes a better thread for the sheet metal screws(drill larger hole in gap seal for screw). I did mine a little different here and just used one tube over the ends of the LE tubes that stick out about 2 inches on my plane.
