Normal Checklist Kolb Firestar N27KF

PRE-FLIGHT

- Cockpit interior: Check controls. Seat belts. Master switch –OFF. Pull pin to parachute.

- Check brake lever operation.

- Check choke and throttle lever operation.

- Windshield integrity. Clean. Latches operate.

- Pitot tube clear.

- Left wheel/brake.

- Left wing strut. Left wing.

- Propeller, check for cracks. Looseness.

- Engine mounting. Wiring. Carb socket. Carb attachment. Exhaust attachment/condition.

- Ailerons; pushrods, hinges.

- Remove and inspect carb fuel bowl.

- Cable ties on parachute bridle.

- Tail wires and pins. All tail surface hinges.

- Tailwheel and springs.

- Right wing strut. Right wing.

- Right wheel/brake.

BEFORE STARTING ENGINE

- Exterior Preflight -- COMPLETE.

- Tach/Hobbs Meter -- RECORD

- Brakes -- TEST & SET parking brake.

STARTING ENGINE

- Pump primer bulb to fill carb.

- Choke set to full.

- Master Switch –- OFF.

- Throttle –- Closed

- Propeller Area -– CLEAR.

- Pull engine through 3-4 times.

- Master Switch -– ON

- Pull engine through until starts.

- Adjust throttle to 3000rpms.

- Reduce choke as engine temp increases.

BEFORE TAKEOFF

- Cabin Doors –- LATCHED.

- Seats, Belts, Harnesses -- ADJUST & LOCKED.

- Door latched.

- Radios & Electrical Equipment – ON.

- Flight Controls -– FREE & CORRECT.

- Brakes – remove parking brake pin.

- Throttle -– 2800 RPM.

- Engine temp –- CHT over 200F.

- Choke - OFF.

8.  Flight Instruments and Radio -- SET.

TAKEOFF

- Normal Takeoff

- Throttle –- FULL OPEN.

- Climb Speed -– 45 mph.

ENROUTE CLIMB.

- Airspeed -– 45 – 55 mph.

- Throttle -– FULL OPEN.

CRUISE

- Power –- 5800 to 6200rpm.

BEFORE LANDING

- Airspeed –- 45-50 mph

NORMAL LANDING

- Touchdown –- MAIN WHEELS FIRST. 35-40 mph.

- Landing Roll -– LOWER TAIL GENTLY.

- Braking -– MINIMUM REQUIRED.

AFTER LANDING

- Unnecessary Equipment -– OFF.

- Stop engine. Master switch – OFF.

SECURING AIRCRAFT

- Parking Brake -– SET (Normally Not Used.)

- Radios, Electrical Equipment –- OFF

- Master Switch –- OFF.

- Control stick secured with seat belt.

- Replace parachute PIN.

- Tach/Hobbs Meter –- RECORD.

- Empty carb fuel bowl.