> I'll give you the same advice that was given to me and that is to use a Volt Ohm Meter and test before you make any assumption. From the Net

> Resistance values of Ducati ignition unit: 
> · Ignition coil - Secondary coil in external box 5.1 to 6.3 ohms 
> · Stator - Light coil, yellow-yellow/black .3 to .35 ohms 
> · Charging coil, green/white 230 to 280 ohms 
>   Rev counter (Tachometer)
> · Pick-up (grey/white) 30 to 35 ohms 
> · Pick-up old type (red-engine ground) to 4017191 50 to 70 ohms 
> · Pick-up, new type (red/white engine ground) from 4017191 140 to 180 ohms 
> Ignition "OFF" - black/yellow-brown (ground) closed circuit
> Ignition "ON" - black/yellow-brown (ground) open circuit